CRYSTAL PARADIGM — SCIDIP FOUNDER BRIEFING

Sona Comstar — IIT Delhi Innovation Program 2026  |  HE-MCGA Project Brief

──────────────────────────────────────────────────────────────────────────────────────────

1. What is the SCIDIP Program?

SCIDIP — Sona Comstar IIT Delhi Innovation Program — is a flagship deep-tech startup incubation program run jointly by FITT IIT Delhi and Sona Comstar. Now in its 6th consecutive edition (Year 2025-26), it runs a national open call targeting startups working on Smart, Safe and Clean mobility technologies.

Key Program Facts:

Total Grant Pool: Rs.1.6 Crore across 5-6 selected startups

Grant per startup: Up to Rs.80 Lakhs as non-dilutive Grant-in-Aid — no equity taken

Incubation duration: 24 months, milestone-linked across 5 funding tranches

Selection: 2 rigorous rounds — Technical Expert Panel, then Techno-Commercial evaluation by investors + industry

Focus Areas: EV Technologies, Advanced Batteries, Advanced Materials, Industry 4.0, Autonomous Vehicles, Sustainability

We have cleared BOTH rounds and are now invited for the final presentation at IIT Delhi

──────────────────────────────────────────────────────────────────────────────────────────

2. Who is FITT IIT Delhi?

FITT — Foundation for Innovation and Technology Transfer — is the official Industry Interface and Technology Commercialization arm of IIT Delhi. It is the primary bridge between IIT Delhi's world-class academic research and real-world industrial deployment.

What FITT Does:

Manages Rs.300+ Crore in R&D contracts with industry annually

Operates NIDHI-CoE, Atal Incubation Centre (AIC) and BioNEST — has incubated 200+ deep-tech startups

Provides access to IIT Delhi CRF: STA (1500°C), LENS/DED, XRD, SEM/EDS — India's most advanced materials characterization cluster

Connects startups to VC/investor network, IP filing cell, Sona Comstar corporate ecosystem

Active aerospace R&D collaborations with DRDO, HAL, ISRO and international OEMs

4 SCIDIP Program Objectives (from official poster):

IIT Delhi & Sona Comstar mentorship from academia, industry experts and selected startup support

Technology & Infrastructure Support: Lab access at IIT Delhi CRF and Sona Comstar facilities

Access to Corporate Market Connect: Network of potential partners, investors and industry experts

Rs.1.6 Cr Total Grant Pool: Prototype development grant to move from concept to physical validation

──────────────────────────────────────────────────────────────────────────────────────────

3. Who is Sona Comstar?

Sona Comstar (NSE: SONACOMS) is India's leading listed automotive systems and components manufacturer — designing, manufacturing and supplying highly engineered, mission-critical automotive systems for both ICE and EV platforms globally.

Elite Facts:

9 manufacturing plants: India, China, Mexico, USA — Rs.3,200+ Crore revenue

Clients: Tata, Mahindra, Ather, Honda, BMW, GM, Ford — top-tier global OEM roster

Core products: Precision driveline gears, differential assemblies, magnet-less Switched Reluctance Motors (SRM), motor control units

EPIC Roadmap: Electric, Personalized, Intelligent, Connected — Sona Comstar's EV future vision

Their H13 forging dies and SRM stator components face the EXACT same thermal failure modes our HE-MCGA material solves

FITT + Sona Comstar Collaboration — Previous Call Winners:

Sona Comstar partners with FITT IIT Delhi because IIT Delhi provides deep materials science infrastructure while Sona Comstar provides the real-world automotive manufacturing testbed. Together they co-fund, co-mentor and co-commercialize selected startups.

Calls 1-5 Winners include: QUANTEON POWERTRAIN, inGO Electric, CREATARA, P3C Technology, MOTOR AMA, Octarange Technologies, Calvem Controls, Aatral ESP, Dopar Energy, Baud Resources, EV Recharge, GroKalp H2CNT, Planzit

Real proof: IGNIT Technical Solutions received Rs.40 Lakhs grant for Battery Thermal Management System under this exact program

──────────────────────────────────────────────────────────────────────────────────────────

4. Why ONLY IIT Delhi? — Not Any Other Institute or Lab

This is the most important point. Our project is a physical materials validation problem requiring the most advanced characterization equipment in India. Here is precisely why IIT Delhi CRF is the only viable option for our top 3 candidates:

Equipment Our Top Candidates Require for TRL 4 Validation:

STA at 1500°C (Simultaneous Thermal Analyzer): Measures oxidation mass gain and thermal stability. Our candidates must clear <10 mg/cm2. Only IIT Delhi CRF has STA at this temperature range in India.

High-Temperature XRD: Physically confirms single-phase BCC stability post-thermal exposure. CALPHAD predicts it — XRD empirically proves zero brittle sigma-phase formation after 1000 thermal cycles.

SEM/EDS Microscopy: Maps compositional gradients layer-by-layer across our 5mm FGM zone. Verifies zero processing voids or macro-cracks after LENS fabrication.

LENS/DED Additive Manufacturing: Physically fabricates our 11-layer FGM witness coupons using our proprietary multi-hopper powder recipe. No private lab in India has multi-hopper LENS capability for refractory HEAs.

High-Temp Tube Furnaces (1600°C): For controlled 1000+ thermal shock cycle testing at real turbine operating temperatures.

Why No Other Option Exists:

Safran (France) — our original collaborator — has already REJECTED the project. They cannot take it further commercially at this stage.

No private lab in India can characterize refractory 9-element RHEA + 11-layer FGM at 1400°C-1600°C. The equipment does not exist outside IIT Delhi CRF.

CSIR-IICT, NITs and IISc do not have the simultaneous combination of STA + LENS/DED + high-temp XRD that our multi-phase validation pipeline requires under one roof.

Our materials science mentor Dr. Kasarapu Venkataramana has worked on DRDO projects at CSIR-IICT and knows exactly which Indian facility can handle this — his answer is IIT Delhi CRF.

IIT Delhi has published 500+ papers on refractory alloys, TBCs, and high-temperature materials — the faculty expertise we need for mentorship exists only there.

──────────────────────────────────────────────────────────────────────────────────────────

5. What Crystal Paradigm Has Built — The HE-MCGA Project

We have computationally engineered an advanced extreme-environment material system: HE-MCGA (High-Entropy Alloy + Multi-layer Ceramic Graded Architecture). This is not theoretical research — it is a fully validated, patent-filed, manufacturing-ready computational platform at TRL 3+.

The Two-Part Architecture:

9-Element Refractory HEA (RHEA): Engineered in the Ta-W-Cr-V-Ti-Hf-Zr-Nb space. Achieves >917.7 MPa yield strength at 1400K with single-phase BCC stability. Exact fractions are patent-protected (App No: 202641084226).

11-Layer FGM: A 5mm power-law gradient (n=1.1) from 100% metal alloy to 100% LZO ceramic. Limits interface shear to 15.95 MPa, preventing delamination under 15,000 RPM centrifugal load.

Transpiration Cooling Network (TCN): Replaces drilled holes (Kt >= 3.0 stress concentrations) with co-deposited micro-porous channels via selective laser sintering. Reduces parasitic bleed air by 15%.

──────────────────────────────────────────────────────────────────────────────────────────

5B. Our Six Engineering Objectives — With Current Status

Every objective has a go/no-go gate drawn from aerospace airworthiness standards:

Objective 1: Structural Safety Factor Certification

Target: 3D Safety Factor >= 1.50 under 15,000 RPM + 1400°C combined loading

Status: ACHIEVED — 3D FEniCS FEA confirmed SF = 1.71, exceeding threshold by 14%

Next: Physical confirmation via IIT Delhi CRF thermal cycling rig + SEM/EDS

Objective 2: Oxidation Kinetics Control

Target: Oxidation mass gain < 10 mg/cm2 at 1400°C

Status: ACHIEVED — MACE + CHGNet MD on 20,000-atom supercell confirmed 7.5 mg/cm2

Next: TGA on physical coupons at IIT Delhi CRF to empirically verify

Objective 3: Phase Stability — Zero Brittle Sigma Phase

Target: Single-phase BCC (VEC = 5.02), zero sigma phase across 873-1173K window

Status: CONFIRMED — CALPHAD TCHEA4 (300K-3000K) + Gradient Boosting R2 = 0.958

Next: High-temperature XRD at IIT Delhi CRF to physically verify BCC retention post thermal cycling

Objective 4: Interface Shear Stress Minimization

Target: Interface shear stress < 20 MPa to prevent FGM delamination

Status: ACHIEVED — Power-law gradient n=1.1 restricts shear to 15.95 MPa

Next: Adhesion + pull-off testing on physically fabricated 11-layer coupons

Objective 5: Strategic Element Substitution (Supply Chain De-risking)

Target: Reduce Hafnium (Hf) by >50% vs. CMSX-4 baseline to eliminate supply chain risk

Status: EXCEEDED — Hf reduced by >75% via Constrained Differential Evolution optimizer (Gorsse 2025 framework)

Additional outcome: 15-20% overall material cost reduction achieved simultaneously

Objective 6: Manufacturing Translation — LENS Recipe Generation

Target: Hardware-agnostic 11-layer FGM manufacturing recipe for multi-hopper LENS 3D printers

Status: COMPLETE — manufacturing_recipe.csv generated: laser power 200-400W, scan speed 500-1000 mm/min, dual-hopper powder flow defined

Next: Execute recipe at IIT Delhi CRF LENS/DED lab to fabricate physical witness coupons

Overall TRL Status:

Current: TRL 3+ — All 6 objectives cleared computationally. Patent filed. Q1-SCI paper published. Manufacturing recipe ready.
SCIDIP Target: TRL 4/5 — Physical fabrication and empirical validation at IIT Delhi CRF.
Post-SCIDIP: TRL 6 — Sona Comstar supply chain integration and commercialization.

──────────────────────────────────────────────────────────────────────────────────────────

6. Expected Founder Questions — Elite Answers

Q: How exactly will you collaborate with IIT Delhi and what is your plan?

A: Phase 1 (Months 1-12 — Shared POC Sprint): We deliver production-ready AI models, multi-physics validation data, and digital twin outputs to IIT Delhi faculty. In return: SCIDIP seed funding tranche + IIT Delhi HPC access for 20,000+ thermal cycle FEA extension runs. Faculty mentorship on designing the physical characterization experimental protocol.

Phase 2 (Months 12-24 — Scale-Out JDA): We provide our 11-layer LENS manufacturing recipe. IIT Delhi CRF physically fabricates FGM witness coupons and runs 1000 thermal cycles at 1400°C using STA, XRD and SEM/EDS. Grant funds: raw powder procurement (Ta, W, Cr, V, Ti, Hf), AM runs and characterization costs. Sona Comstar connects us to their precision forging and EV supply chain.

Exit at 24 months: Fully validated TRL 5/6 hardware handed over to Sona Comstar for integration into H13 forging dies and SRM stator components.

Q: Why should Crystal Paradigm — an AI startup — be trusted to deliver physical materials validation?

A: We are not a pure software company. Our team is a rare cross-disciplinary combination:

Dr. Kasarapu Venkataramana (12+ years hands-on, CSIR-IICT DRDO projects) will lead the physical lab work at IIT Delhi — he has personally synthesized and characterized nano-composite electrolytes and fuel cell materials.

Dr. Shilpa Choudhary (Stanford Top 2% Scientist, 18+ years AI, 100+ publications, multiple patents) anchors the computational platform.

Our AI team has a Q1-SCI paper (Next Materials, 2026) and Indian Patent No. 202641084226 filed. The SCIDIP evaluation committee — IIT Delhi faculty, industry experts, investors — has already cleared us through two rigorous rounds. That is the most credible third-party validation available.

Q: What happens after 24 months — what is the commercial exit?

A: 3 parallel tracks:

1. LICENSING: License our 11-layer FGM recipe and HEA composition to aerospace OEMs (Safran, GE Aerospace, Rolls-Royce) under technology licensing agreements. Patent provides the IP moat.

2. SONA COMSTAR SUPPLY CHAIN: Validated FGM prototypes integrated directly into Sona Comstar's precision forging dies and SRM stator components — the exact industrial pathway SCIDIP is designed to create.

3. CRYSTAL PARADIGM PLATFORM EXPANSION: Our AI inverse design platform is sector-agnostic. Same platform applies to grid-scale battery materials (Q1 paper already published), EV thermal management, and MSME industrial tooling. IIT Delhi incubation gives credibility to pursue DST, NITI Aayog and BIRAC grants for parallel tracks.

──────────────────────────────────────────────────────────────────────────────────────────

Crystal Paradigm Private Limited  |  crystalparadigm.in  |  khub.crystalparadigm@gmail.com